Architecture Roadmap

The World's First Voice-Operated Agentic TV Platform

A MyPhone Family Product  ·  Version 9.0  ·  Confidential

# 1. Executive Summary

MyTV is an AI-powered television platform that turns any large-format display into an intelligent, voice-controlled room computer.

MyTV is a sister product to MyPhone, sharing the same philosophy of privacy, offline capability, and user control. Where MyPhone is the secure smartphone for the individual, MyTV brings that same intelligence to the shared living space — a unified device that replaces three products: a media system, a video conference station, and a workstation, all controlled by natural voice through an AI agent running entirely on-device.

The product model is deliberately simple. The large-format display has multiple HDMI inputs. Users connect their existing cable boxes, satellite receivers, and other devices to those inputs exactly as they always have. MyTV occupies one HDMI input and contributes what no external device can: a deep voice-control layer, a unified media library, end-to-end encrypted video calling, retro gaming and PC game streaming, and seamless integration with MyPhone.

| Dimension | Specification |
| --- | --- |
| Platform | Raspberry Pi 5 8GB · Hailo-8L 13 TOPS NPU · 256GB NVMe · Infineon SLB9670 hardware TPM · Argon ONE V3 enclosure |
| Voice pipeline | Hybrid intent architecture: fine-tuned MiniLM-L6 classifier for 90% of commands (~605ms end-to-end). Gemma 2 2B IT INT4 (Google DeepMind) for complex queries (~2,100ms). Whisper Small INT8 on Hailo-8L NPU for speech-to-text. |
| Microphone | Blue Yeti Nano (fixed installation, hardware mute, PipeWire WebRTC AEC) or Rode Wireless ME (large room, body-worn, natural 25-30dB echo rejection + PipeWire AEC). |
| Media library | Custom MyTV Media Center — first-party unified library (audio, video, photos, documents). Three-tier encrypted access: USB local · MyPhone Cast (TLS 1.3 + device cert) · MyCloud (client-side E2EE). |
| Video calling | Matrix-native. 1:1 via Matrix VoIP (E2EE via Olm/Megolm). Group via Element Call (MSC3401, E2EE via Megolm). Served from MyCloud. Requires subscription. |
| Gaming | RetroPie retro gaming (NES–PS1). RetroArch Netplay for cross-device multiplayer (MyTV + MyTV or MyTV + MyPhone via MyCloud App Store companion app). |
| Security | Hardware TPM 2.0 disk encryption (PCR-bound LUKS). Signed A/B OTA updates (MyTV OTA Firmware Signing Key, 2-of-3 quorum, subordinate to MyCloud Root CA). MyCloud device certificate (MTV- namespace, issued by MyCloud Device Provisioning CA). Suspend-to-RAM primary power mode (2s resume). nftables default-deny firewall. AppArmor application sandboxing. |
| MyPhone integration | Screen cast (WebRTC, MyCloud device cert auth, 1080p30). Matrix E2EE call handoff. Shared media library via MyPhone Cast (Tier 2) and MyCloud direct (Tier 3 — no MyPhone required). Phone-as-remote via MyTV companion app. Presence-based profile unlock (opt-in mDNS beacon). |
| Subscription | MyTV box operates fully standalone for media, gaming, and workstation functions. MyCloud subscription (the MyCloud subscription (see current pricing at mycloud.com)) unlocks: Matrix E2EE video calling, encrypted media cloud sync, and cross-device continuity features. |

# 2. Product Positioning

MyTV delivers what no commercial smart TV platform provides: a complete intelligence layer with deep local AI, end-to-end encrypted communication, and a unified voice-controlled experience that works without the cloud.

## 2.1 What MyTV Does Exceptionally Well

- Local media at full quality — 4K hardware decode (H.264, H.265, VP9) via Pi 5 VideoCore VII. Unified offline-first library covering all media types. No subscription required for local content.

- YouTube and non-DRM streaming — VP9 hardware-decoded up to 4K via Chromium. YouTube, Twitch, Plex, and all non-DRM streaming services at full quality.

- End-to-end encrypted video calls — Matrix VoIP for 1:1 calls. Element Call for group calls. Megolm encryption throughout. Served from MyCloud — no third-party server has access to call media. Requires subscription.

- Retro gaming and PC game streaming — RetroPie covers the classic gaming catalogue from NES through PlayStation 1 at 1080p upscaled. RetroArch Netplay enables cross-device multiplayer with another MyTV or a paired MyPhone.

- Voice control depth — Every function is voice-accessible via the "Hey MyTV" wake word. The hybrid intent classifier makes common commands respond in under one second. Context window enables multi-turn conversation.

- MyPhone ecosystem integration — Screen cast, call handoff, shared media library, phone-as-remote, and presence-based profile switching — features no commercial smart TV platform offers.

- Display input switching — MyTV voice commands switch between all HDMI inputs on the display. "Hey MyTV, switch to cable" activates while any input is showing. Users keep all their existing devices.

## 2.2 Platform Limitations — Documented Honestly

Platform constraint: The Raspberry Pi 5 supports Widevine L3 content protection only. Netflix, Disney+, and Amazon Prime Video require Widevine L1 for HD and 4K content. With L3, these services deliver SD quality only. This is a Broadcom BCM2712 hardware constraint that no software update can resolve.

MyTV's strength is the combination of capabilities it delivers that no dedicated streaming device provides: full local media library, deep voice control, encrypted video calling, gaming, workstation access, and MyPhone integration. Users who require HD streaming from DRM-protected services connect those devices to the display's other HDMI inputs — which they already own and can switch to by voice.

- YouTube 4K — VP9 hardware-decoded to 4K via Chromium with codec flags. AV1 (YouTube's default 4K codec) is software-only on Pi 5 VideoCore VII; VP9 is forced via launch flags. Visually indistinguishable from AV1 at streaming bitrates.

- AAA gaming — Not native to Pi 5 GPU. The product family does not support PC game streaming services. RetroPie covers the classic gaming catalogue.

# 3. Hardware Stack

Figure 3.1 — MyTV system architecture. Hardware layer (top), software stack (centre), services layer (bottom). Three operating modes: TV, Conference, Workstation.

## 3.1 Core Compute Platform — Raspberry Pi 5

| Component | Specification | Role in MyTV |
| --- | --- | --- |
| CPU | ARM Cortex-A76 quad-core 2.4GHz (overclockable to 3.0GHz with active cooling) | OS, agent system, MiniLM-L6 classifier, Gemma 2 2B inference, application layer |
| RAM | 8GB LPDDR4X — mandatory minimum. zram compression (4GB configured) yields ~12GB effective headroom. | Gemma 2 2B INT4 requires ~1.5GB. Remaining headroom for OS, media playback, conference client. |
| GPU / display output | VideoCore VII. Dual HDMI 2.0. 4K60 on HDMI 0. Hardware decode: H.264, H.265, VP9. | 4K local media decode in hardware. AV1 decode is software-only — VP9 forced for YouTube. |
| Storage | 256GB NVMe SSD via PCIe M.2 HAT (sole PCIe device). Hailo-8L on USB3 (separate). | OS (A/B OTA partitions), user media library, AI models, application data. |
| Connectivity | Wi-Fi 5 (802.11ac), Bluetooth 5.0, 2× USB 3.0, 2× USB 2.0, Gigabit Ethernet. | Gigabit Ethernet recommended for media-intensive installs. Wi-Fi 5 adequate for most home use. |
| Power | Official Raspberry Pi 27W USB-C PD PSU (5V/5A). Mandatory. | Standard 3A USB-C PSUs insufficient for Pi 5 + Hailo USB3 + active cooling under sustained load. |

## 3.2 Hailo-8L NPU — Speech Recognition Accelerator

The Hailo-8L AI HAT+ connects via USB3 (Hailo USB3 SDK). It runs Whisper Small INT8 only — it is a CNN and vision accelerator, not an LLM engine. The large language model (Gemma 2 2B) and intent classifier (MiniLM-L6) run on the Pi 5 ARM CPU via llama.cpp NEON optimisation. This architecture is the correct one: Whisper is a valid Hailo Model Zoo entry with hardware-accelerated inference; LLMs require autoregressive decoder attention that the Hailo-8L does not support.

| Model | Hardware | Latency | Role |
| --- | --- | --- | --- |
| openWakeWord | Pi 5 CPU (single core) | <5ms | Always-on "Hey MyTV" detection. <0.1 false trigger per hour. |
| Whisper Small INT8 | Hailo-8L NPU via USB3 | ~400ms | Speech-to-text transcription of user utterance. |
| MiniLM-L6 classifier | Pi 5 CPU · 22MB | <50ms | Fast intent classification. 15 classes. Handles 90% of commands. |
| Gemma 2 2B IT INT4 | Pi 5 CPU · llama.cpp NEON · 1.5GB | ~1,600ms | LLM fallback for complex, ambiguous, or multi-turn queries (10% of commands). |
| Piper TTS | Pi 5 CPU | <100ms | Natural spoken confirmation after each executed command. |

## 3.3 Camera — Leopard Imaging LI-IMX179-USB-130H-CONN

- Sensor — Sony IMX179 · 8MP · 130° diagonal FOV · USB 3.0 UVC · plug-and-play on Linux · no driver required

- Video modes — 4K@15fps · 1080p60 · 720p60 · MJPEG hardware compression

- Conference use — 130° wide-angle captures the full room without pan or tilt hardware. Appropriate for 2–6 person meeting rooms and living rooms.

- Mounting — Top-of-display via 1/4-20 tripod thread or bezel clip. 1.8m active USB3 extension cable for installations where the Pi 5 enclosure is rack-mounted or behind the display.

## 3.4 Microphone

| Device | Best for | Echo cancellation | Hardware mute |
| --- | --- | --- | --- |
| Blue Yeti Nano | Fixed desk / media console. Cardioid polar pattern faces user; rejects TV speaker direction passively. | PipeWire WebRTC AEC module. HDMI ARC audio output as loopback echo reference. Effective at TV levels up to ~75dB at 2m. | Dedicated hardware button with LED indicator (red = muted). |
| Rode Wireless ME | Large rooms, mobile users. Body-worn transmitter 15-20cm from mouth; TV speaker 2-3m away. | Natural acoustic rejection ~25-30dB from body-worn proximity (primary). PipeWire AEC secondary layer. | Physical mute button on transmitter body. 30m wireless range. |

PipeWire AEC configuration is identical for both microphones — a single config file routes either USB source through the WebRTC echo-cancel module with the HDMI ARC output as the speaker reference signal.

## 3.5 Hardware Security — Infineon SLB9670 TPM 2.0

A Waveshare TPM2.0 Module (Infineon SLB9670) connects via SPI GPIO header. This is a hardware TPM 2.0 chip — the same Infineon family as the MyPhone OPTIGA security element. It provides tamper-resistant key storage for the LUKS full-disk encryption key, which is sealed against TPM PCR values representing the verified boot chain. Extracting the NVMe SSD and connecting it to a different system will not yield the LUKS key without the TPM.

## 3.6 Compatible Displays

MyTV is designed and validated for NEC MultiSync and Sharp NEC large-format commercial displays. These are preferred for 24/7 operational reliability, quality HDMI-CEC implementations, and the MA/P series MPi4E variants that include an on-panel Raspberry Pi Compute Module slot.

| Series | Sizes | Key capabilities | Recommended configuration |
| --- | --- | --- | --- |
| M Series | 43"–98" | 4K UHD · HDMI 2.0 (multiple inputs) · HDMI-CEC · RS-232 control · 24/7 operation rated | M551 (55") for home living room. M651 (65") for larger spaces. M751/M861/M981 for commercial conference rooms. |
| MA Series | 43"–55" | All M features + optional IR touchscreen · Raspberry Pi compute module slot (-MPi4E variant) | MA431-MPi4E: compact 43" installation with Pi slot. External Pi 5 enclosure used for v1 (Pi CM4 currently in slot; Pi CM5 support planned). |
| P Series | 43"–55" | Premium commercial grade · 700cd/m² · IR touch · Pi slot · Precision touch variants | P555-MPi4E: highest quality MyTV display. Future single-cable display+compute unit when Pi CM5 available for the MPi4E slot. |

Note: All NEC/Sharp M, MA, and P series displays have multiple HDMI inputs. Users connect all their existing external devices to those inputs unchanged. MyTV occupies one input and voices-commands can switch between any input: "Hey MyTV, switch to HDMI 2."

# 4. Voice Pipeline

From "Hey MyTV" to completed action in under one second for 90% of commands. All inference on-device. No audio sent to any server.

Figure 4.1 — Voice pipeline end-to-end. Two microphone paths converge after PipeWire AEC. MiniLM-L6 fast classifier for 90% of commands (~605ms). Gemma 2 2B IT INT4 from Google DeepMind for complex queries (~2,100ms).

## 4.1 Pipeline Stages

- openWakeWord continuously monitors the microphone stream on a single CPU core. Detects "Hey MyTV" in under 5ms with fewer than 0.1 false triggers per hour in a normal home environment.

- PipeWire WebRTC AEC subtracts the TV speaker signal from the microphone input using the HDMI ARC output as a loopback reference. Both the Yeti Nano and Rode Wireless ME route through the same AEC virtual device. Echo cancellation is effective at TV audio levels up to ~75dB.

- Whisper Small INT8 transcribes the utterance on the Hailo-8L NPU (USB3 mode) in approximately 400ms. Real-time factor ~1.0. Output: a UTF-8 text string.

- The MiniLM-L6 intent classifier evaluates the transcribed text against 15 intent classes in under 50ms on CPU. If confidence exceeds 0.85, the intent routes directly to the agent. If below threshold, it falls through to Gemma 2.

- Gemma 2 2B IT INT4 (Google DeepMind, 1.5GB RAM) handles complex, ambiguous, or multi-turn queries via llama.cpp with ARM NEON optimisation. Produces a JSON function call in approximately 1,600ms. A "thinking…" indicator appears on screen within 500ms of invocation.

- The agent router validates the JSON schema, executes the action via D-Bus, and speaks a confirmation via Piper TTS within ~100ms of execution. The user hears the result approximately 605ms after finishing their sentence for common commands.

## 4.2 Hybrid Intent Architecture — Why Not a Single LLM

90% of television voice interactions come from a closed vocabulary: "play [title]," "tune to [channel]," "start a meeting," "volume up," "switch to HDMI 2." A 22MB sentence transformer fine-tuned on 5,000 annotated TV command utterances classifies these in under 50ms on CPU. Running a 1.5GB language model for these interactions would add 1,600ms of latency with no benefit.

The LLM handles the 10% where genuine language understanding is required: ambiguous entity resolution ("the Batman one" referencing a previous search), multi-entity extraction ("find a comedy from the 80s with Eddie Murphy"), and conversational context where the previous turn must be interpreted. A 60-second context window stores the previous intent and response so that follow-up commands resolve correctly.

## 4.3 Agent System

| Agent | Responsibilities |
| --- | --- |
| Media Agent | Search and playback across the unified MyTV Media Center library. Source routing across USB, MyPhone Cast, and MyCloud. Playlist and queue management. MyPhone media library access. |
| Conference Agent | Matrix VoIP 1:1 calls. Element Call group calls. Call mute, unmute, camera, screen share, end. CalDAV calendar auto-join. Matrix call handoff from MyPhone. All call state controlled via D-Bus interface to native Matrix client. |
| Games Agent | RetroPie local launch. RetroArch Netplay session hosting and management. Cross-device multiplayer invitations via Matrix E2EE messages. Voice command: “Hey MyTV, start retro multiplayer with [name].” |
| System Agent | Volume. Display brightness via RS-232. Operating mode switching (TV, Conference, Workstation). Settings navigation. Suspend/wake. OTA update trigger. |
| Input Agent | HDMI-CEC display input switching: "Hey MyTV, switch to HDMI 2," "Hey MyTV, switch to cable," "Hey MyTV, come back." Active regardless of which display input is currently showing. |
| MyPhone Bridge Agent | Screen cast receive (WebRTC DTLS-SRTP). Matrix E2EE call handoff coordination. MyPhone media library source management. Phone-as-remote service. Presence-based profile unlock. |

## 4.4 Safety and Robustness

- Destructive action gate — Actions categorized as destructive (delete, format, factory reset, unpair) require a secondary voice confirmation — "Hey MyTV, confirm" — within 10 seconds. Protects against adversarial voice injection from TV audio.

- Voice failure recovery — Three-attempt hierarchy: (1) re-prompt; (2) on-screen "Did you mean?" suggestions from the classifier's nearest matches; (3) automatic fallback to remote control mode. Users are never left in a dead state.

- Context window — 60-second conversation context. "Find Batman movies" → "Play the 1989 one" resolves correctly without restating the subject.

# 5. Software Stack

## 5.1 Operating System and Compositor

- OS — Raspberry Pi OS Bookworm (64-bit Debian 12). Required for Hailo-8L NPU support and Gemma 2 2B INT4 memory addressing. 64-bit build only.

- Compositor — Labwc (wlroots-based Wayland compositor). Lightweight, fast startup, full Pi 5 VideoCore VII driver support. Presents the TV-mode 10-foot UI shell on startup.

- Audio system — PipeWire handles microphone routing, HDMI ARC output, WebRTC AEC echo cancellation, and application audio mixing. A single PipeWire configuration file routes both Yeti Nano and Rode Wireless ME through the same AEC virtual device.

- Boot time — Cold boot target: 22 seconds. Suspend-to-RAM (S2RAM) is the primary power-off mode — 2 second resume. Power button triggers S2RAM. Full cold boot is reserved for OTA update installation.

- Security hardening — nftables firewall (default-deny inbound). AppArmor profiles for all media and conference applications. SSH disabled in production firmware. No root login. Minimum exposed network surface.

## 5.2 Display Input Model

MyTV occupies one HDMI input on the NEC/Sharp display. The display's remaining inputs are available for any external devices the user chooses to connect — exactly as if MyTV were not present. MyTV does not intercept, manage, or record signals on those inputs.

| Input | Source | Managed by |
| --- | --- | --- |
| HDMI 1 (example) | MyTV (Raspberry Pi 5) | MyTV software and voice agent. Native applications, MyPhone integration. |
| HDMI 2, 3… (example) | Any external device: cable box, satellite receiver, games console, streaming device | The external device and its own controls. MyTV does not touch these inputs. |
| HDMI ARC | Audio return channel (all inputs share this for speaker output) | Shared. MyTV uses HDMI ARC audio as the echo reference for its AEC pipeline. |

The Input Agent enables voice switching between all inputs. "Hey MyTV, switch to cable" sends a CEC command to the display to activate that input. "Hey MyTV, come back" returns to the MyTV input. The voice pipeline continues listening regardless of which input is active — so the user can always call MyTV back by voice.

## 5.3 Operating Modes

| Mode | How to activate | UI optimisation | Primary input method |
| --- | --- | --- | --- |
| TV Mode | Default on startup. "Hey MyTV, TV mode." | 10-foot interface. Large fonts, high contrast, d-pad navigation. MyTV Media Center as the home screen. | Voice (primary), HDMI-CEC remote, Rii keyboard, MyPhone companion app remote. |
| Conference Mode | "Hey MyTV, call [name]" or "start a meeting." Calendar auto-join. | Fullscreen video tile layout. Self-view. Mute and camera status indicators. | Voice for all call control. Keyboard for text chat. |
| Workstation Mode | "Hey MyTV, workstation." Connecting keyboard and mouse. | Standard desktop with taskbar, windowed applications, and mouse cursor. | Keyboard and mouse primary. Voice for application launch and search. |

# 6. Native Applications

## 6.1 MyTV Media Center

A first-party unified offline media library. Same design philosophy as the MyPhone Media Center — one library, all media types, no cloud dependency for local content.

| Pillar | Supported formats | Decode engine | TV-optimised features |
| --- | --- | --- | --- |
| Audio | MP3, FLAC, AAC, OGG, OPUS, WAV, M4A, AIFF | FFmpeg / GStreamer pipeline | Background playback with lock-screen transport controls. Smart playlists. Podcast engine (RSS, chapter support, auto-download). Equalizer. 10-foot artist and album browser. |
| Video | MP4, MKV, WebM, AVI, MOV, H.265/HEVC, VP9 | FFmpeg + VideoCore VII HW decode | Auto-thumbnail generation. Subtitle support (SRT, ASS, VTT). Resume playback. Series and collection grouping. Picture-in-picture mode. |
| Photos | JPG, PNG, HEIC, WebP, RAW (DNG, ARW, NEF, CR2) | libraw + ImageMagick + gdk-pixbuf | Timeline and album views. TV slideshow mode. EXIF viewer. Non-destructive edits: crop, rotate, exposure, colour correction. |
| Documents | PDF, EPUB, MOBI, TXT, Markdown, HTML | poppler (PDF), foliate (EPUB), Pandoc | Full-screen reader mode with large typography for TV viewing distance. PDF annotations and bookmarks. eBook reading progress synced with MyPhone. |

- Unified library engine — Single SQLite database with FTS5 full-text search indexes all four media types. A single voice query — "Hey MyTV, find anything about space" — searches across movies, albums, documents, and photos simultaneously.

- Import pipeline — USB drive auto-mount and index on insertion. yt-dlp integration for downloading audio and video to the local library offline (not streaming — downloaded to the device). Import daemon generates metadata and thumbnails automatically.

## 6.2 Conference — Matrix-Native Video Calling

End-to-end encrypted video calling. 1:1 and group. Served from MyCloud. The same Matrix protocol used by MyPhone — no second video system, no bridge required.

The Conference application uses a native Linux Matrix client built on the Matrix Rust SDK. This provides clean D-Bus interfaces for voice control — the Conference Agent calls org.myphone.matrix.call.mute(), .hangup(), and .camera_toggle() directly. No browser kiosk, no JavaScript injection, no fragile accessibility workarounds.

IMPORTANT — EMERGENCY CALLING  MyTV Matrix VoIP does not support emergency calling (E911 / 112 / 999). For emergencies, use a cellular phone, a landline, or any PSTN-connected device. Ensure a cellular device is accessible in any room where MyTV is the primary communication device. This limitation applies to all Matrix VoIP calls regardless of subscription status.

| Call type | Protocol | E2EE | Requires |
| --- | --- | --- | --- |
| 1:1 video call | Matrix VoIP (WebRTC signalled via Matrix). DTLS-SRTP for media. | Olm key exchange. The MyCloud server relays signalling only — it cannot see media. | MyCloud subscription |
| Group video call | Element Call (Matrix MSC3401). LiveKit-based SFU on MyCloud. | Megolm group encryption. The SFU forwards encrypted packets without decrypting. Server cannot see call media. | MyCloud subscription |
| Call handoff from MyPhone | "Hey MyTV, take the call." MyTV joins the active Matrix room using its own MyCloud device identity (MTV- class, registered with MyCloud Conduit). E2EE preserved throughout. MyPhone remains as a second participant. | E2EE preserved throughout. MyTV joins as a second device identity. MyPhone remains in the room. | Both devices registered on MyCloud |
| External participants | Matrix federation: any Matrix user on any homeserver can be invited by their Matrix ID. | E2EE maintained for all federated participants. | Participant has a Matrix account (any server) |

CalDAV calendar integration enables automatic meeting join. Calendar events with Matrix room links generate a "Your 2pm call starts in 5 minutes" voice announcement, and "Hey MyTV, join the meeting" or automatic join on confirmation.

## 6.3 Retro Gaming

### 6.3.1 Mode 1 — Solo Retro (RetroPie)

RetroPie with EmulationStation provides a curated retro gaming hub running entirely on-device. The Pi 5 at 3.0GHz (overclocked with active cooling) handles NES through PlayStation 1 emulation smoothly, with N64 games running at 1080p upscaled. ROM files are purchased through the MyCloud App Store. Controller support via the Linux HID input system for USB and Bluetooth gamepads with automatic RetroArch button mapping. No networking required. Voice command: "Hey MyTV, play [game name]."

### 6.3.2 Mode 2 — Play Together (RetroArch Netplay)

RetroArch Netplay is the single cross-device multiplayer protocol for the MyPhone product family. It is fully open source and auditable, requires no third-party cloud infrastructure, and transmits only controller input and emulator state — not video. Each device renders the game locally. Session invitations travel via Matrix E2EE. MyCloud provides a lightweight Netplay relay service for internet-based sessions when both devices are not on the same local network (subscription required for relay).

| Mode | How it works | Voice command |
| --- | --- | --- |
| MyTV + MyTV | Both MyTVs run the emulator independently. Netplay synchronises controller input and emulator state. Each TV renders locally — no video streaming, minimal bandwidth. Session invitation sent as Matrix E2EE message. | "Start retro multiplayer with Alex" — Games Agent hosts a Netplay session and delivers the host address to Alex via Matrix. |
| MyTV + MyPhone | MyTV is the Netplay host. Paired MyPhone joins using the MyTV companion app (installed from the MyCloud App Store). MyPhone sends controller input only — the game renders on MyTV. Invitation delivered via Matrix E2EE. | "Invite Alex to play on their phone" — Games Agent hosts Netplay, sends session link to Alex’s MyPhone via Matrix. |

SECURITY NOTE  RetroArch Netplay transmits only controller input and emulator state synchronisation packets — it does not stream video between devices. This is the architecturally correct cross-device gaming protocol: the attack surface is minimal (no screen content exposure, no third-party infrastructure), and both MyTV and MyPhone render independently. Third-party game streaming services (Parsec, Steam Remote Play, Xbox Cloud) are not supported and have been removed from the architecture. They introduce third-party cloud dependencies and expand the attack surface in ways inconsistent with the product family’s security model.

## 6.4 Browser

- TV mode — Chromium kiosk with a large-button streaming service launcher page. VP9 forced via launch flags for YouTube 4K hardware decode. d-pad navigation optimised for remote control.

- Workstation mode — Full Chromium with uBlock Origin and Privacy Badger pre-installed. DNS-over-HTTPS to Cloudflare 1.1.1.1. No Google account sync by default.

Note: Chromium is an on-demand application in v1, not a permanent resident process. This frees approximately 300-500MB RAM when the browser is not in use — meaningful headroom on an 8GB device running Gemma 2 2B.

# 7. Media Access Architecture

Three-tier encrypted architecture. DLNA and SMB are intentionally absent — they provide no authentication and no encryption. MyPhone Cast delivers the same capability — encrypted, authenticated, seekable media streaming — using infrastructure already present in both products.

Figure 7.1 — Three-tier media access. Tier 1: USB (direct, always available). Tier 2: MyPhone Cast (mDNS + HTTP/2 + TLS 1.3 + device cert mutual auth). Tier 3: MyCloud (client-side E2EE, SPU key derivation). MyPhone is the encryption authority for cloud-stored media.

## 7.1 Tier 1 — USB Local Storage

A USB drive plugged into the Pi 5 is auto-mounted and indexed by the MyTV Media Center. Supports FAT32, exFAT, NTFS, and ext4. The import daemon fingerprints new files, extracts metadata via FFmpeg and libraw, generates thumbnails, and populates the SQLite library within approximately 30 seconds of drive insertion. Voice: "Hey MyTV, eject the USB drive" performs a clean unmount before physical removal.

## 7.2 Tier 2 — MyPhone Cast (encrypted local network)

When MyPhone joins the same Wi-Fi network as a paired MyTV, it advertises a secure media server via mDNS (_myphone-cast._tcp) and opens a local HTTPS server on port 8443. MyTV discovers this service and connects using mutual TLS 1.3 with device certificate authentication — both devices present their device certificates, and MyPhone verifies MyTV's certificate against its paired device list. Unknown devices are refused before any media is served.

The API uses HTTP/2 with Range request headers, enabling arbitrary seeking in large video and audio files without buffering the full file. First-frame latency is approximately 50ms on a local 5GHz Wi-Fi network. A WebSocket event channel delivers real-time library change notifications — a photo taken on MyPhone appears in the MyTV Media Center within seconds.

| Property | DLNA (replaced) | MyPhone Cast |
| --- | --- | --- |
| Encryption in transit | None — all media served in plaintext | TLS 1.3 unconditionally. All media bytes encrypted. |
| Authentication | None — any device on the network can browse and stream | Mutual device certificate auth. MyPhone verifies caller cert against its paired device list. |
| Integration with device certs | None | Full. Uses the device certificate infrastructure already built for MyPhone. |
| Forward secrecy | N/A | TLS 1.3 mandates ephemeral key exchange. Past sessions cannot be decrypted if device keys are later compromised. |
| Seeking in large files | Limited and unreliable | Native HTTP Range headers. Arbitrary byte-range seeking. Same technology CDNs use for video. |

## 7.3 Tier 3 — MyCloud (encrypted sync)

The cloud tier enables MyTV to access MyCloud-synced media directly — when MyPhone is not on the local network, or when there is no paired MyPhone at all. MyTV authenticates to MyCloud using its MyCloud device certificate and decrypts media using the Layer A MyCloud Account Key (see MyCloud Architecture Roadmap Section 5.1). No MyPhone presence is required for Tier 3 access. This is the correct architecture for a device-independent cloud service: a MyTV owner without a MyPhone has equal access to their MyCloud-synced library.

- Client-side encryption — Every file is encrypted by the MyPhone SPU before upload. Key derivation path: m/3/0 via the cryptod DERIVE_KEY operation. Algorithm: AES-256-GCM. The server holds only ciphertext. The MyCloud server cannot decrypt any media file.

- Decryption authority — MyTV downloads cloud-stored files and requests decryption keys from MyPhone Cast. MyPhone derives and returns keys only to paired devices. If MyPhone is completely offline, MyTV cannot decrypt cloud-stored media — the SPU remains the encryption authority.

- Sync policy — Photos and documents auto-sync (typically under 50GB). Audio and video are user-selected playlists only — file sizes make automatic sync impractical. Library metadata (album structure, playlists, reading progress, thumbnails) always syncs and is small.

- Family use case — Photos taken on a family member's MyPhone appear on the home MyTV within minutes via the cloud, even when that person is not home. The family photo library is always current on the living room display.

## 7.4 Source Router

The MyTV Media Center source router automatically selects the best available source for each media item. From the user's perspective, the library is a single unified collection — they say "play my Venice photos" and the system finds them on whichever tier has them.

# Source priority (simplified)
for each media request:
  if file present on USB drive         → stream from USB   (fastest, no network)
  elif MyPhone reachable on local Wi-Fi → stream via Cast   (encrypted, ~50ms)
  elif file synced to MyCloud      → stream from cloud (E2EE, requires subscription)
  else                                  → show unavailable indicator

# 8. Security Model

MyTV is a shared room device. Its security model is calibrated for that threat model — not the hardware enclave model of MyPhone, and not naively under-specified. The fundamental principle: MyTV never stores wallet keys, biometric data, or any MyPhone TCB secrets.

## 8.1 Verified Boot and Hardware Disk Encryption

- Pi 5 BootROM validates the EEPROM bootloader against the OTP-burned MyTV Boot Key (the MyTV OTA Firmware Signing Key public key, stored in the Thales Luna HSM under the MyCloud Root CA, 2-of-3 quorum — see MyCloud Architecture Roadmap Section 3).

- The bootloader validates the Linux kernel. The kernel hash is extended into the Infineon SLB9670 hardware TPM PCR 0.

- systemd-cryptenroll unseals the LUKS disk encryption master key from the hardware TPM against the expected PCR values. On the verified boot path: disk unlocks and the OS starts. On any deviation (tampered bootloader, different kernel, NVMe extracted to another machine): PCRs do not match, disk remains locked.

- The Data partition (user media, application data, AI models) is encrypted separately from the root filesystem. OTA updates overwrite Root-A or Root-B but never touch the Data partition.

## 8.2 OTA Updates — Signed A/B Partitions

NVMe partition layout: Boot (256MB FAT32), Root-A (30GB ext4), Root-B (30GB ext4), Data (remaining ext4). One root partition is active; the other is standby for OTA writes. OTA packages are signed by the MyTV OTA Firmware Signing Key (2-of-3 HSM quorum, subordinate to the MyCloud Root CA). Full key hierarchy and ceremony specification: see MyCloud Architecture Roadmap Section 3. APT verifies signatures before installation.

On OTA update: the signed package is written to the standby root partition, the boot flag is updated, and the device reboots. If the new partition fails to boot after 3 attempts, the boot flag reverts to the previous working partition. Users never lose a working system.

## 8.3 Network Security

| Port | Protocol | Service | Scope | Notes |
| --- | --- | --- | --- | --- |
| 5353 | UDP | mDNS — presence detection, MyPhone Cast discovery, remote control service | Local subnet | Existing service, no change. |
| 8443 | TCP | MyPhone remote control API and Cast client inbound from paired MyPhone | Local subnet | Device certificate mutual auth required. Non-paired devices refused. |
| 443 | TCP | Outbound: Matrix client to MyCloud. MyCloud media download. Netplay relay to MyCloud relay service (UDP, subscription required for internet sessions). | Outbound only | All outbound HTTPS. No inbound ports required for these services. |
| All others |  | No SSH, no VNC, no Telnet, no other services. | Blocked | nftables default-deny inbound. Attack surface minimised. |

## 8.4 Application Sandboxing

- AppArmor profiles — All media and conference applications run under AppArmor profiles that restrict filesystem access to declared paths.

- HDMI-CEC whitelist — CEC commands from external HDMI devices are whitelisted: volume control, d-pad navigation, power, and input switching. Arbitrary CEC commands from non-display sources are blocked. Prevents a malicious HDMI device from manipulating MyTV.

## 8.5 MyPhone Pairing Revocation

Settings → Paired Devices lists all paired MyPhone devices. Removing a device revokes its certificate from MyTV's trust store immediately. Both MyPhone and MyTV device certificates are issued by the MyCloud Device Provisioning CA under namespace-separated hierarchies (MP- for MyPhone devices, MTV- for MyTV devices). Remote revocation via MyCloud propagates to all paired devices within 60 seconds of a loss report via MyCloud's OCSP endpoint and delta CRL. Offline MyTV devices receive the revocation on next MyCloud connection, queued until delivery is confirmed. See MyCloud Architecture Roadmap Section 4.4 for the full revocation specification.

## 8.6 Security Boundary — MyTV and MyPhone TCB

Architectural boundary: MyPhone's seL4 TCB (cryptod, walletd, biometricd) is never accessible from MyTV over any interface. The Bitcoin wallet private keys and biometric templates live in the MyPhone SPU hardware and never leave it. Cross-product features use content signals (screen pixels, media metadata) and presence signals (device certificate identity) only. MyTV cannot perform cryptographic operations on the user's behalf as a proxy for MyPhone.

# 9. MyPhone + MyTV Ecosystem

The MyPhone product family delivers cross-device capabilities that no commercial product ecosystem matches. End-to-end encrypted call handoff from phone to TV — a live call moves from your pocket to the room without dropping, fully encrypted throughout. Presence-based profile switching that activates your profile when your phone arrives home. Gaming invitations that travel through your private encrypted message channel, not a third-party service. These capabilities exist because MyPhone and MyTV are built on the same identity, encryption, and communication infrastructure — MyCloud — rather than assembled as an afterthought.  All real-time features use local Wi-Fi when both devices are on the same network. No cloud relay for latency-sensitive operations.

Figure 9.1 — MyPhone + MyTV cross-product integration. All real-time features use local Wi-Fi. MyCloud handles backup, sync, and OTA only. MyPhone seL4 TCB never accessible from MyTV.

| Feature | How it works | Network requirement |
| --- | --- | --- |
| Screen cast | MyTV advertises a WebRTC casting service via mDNS. MyPhone discovers it automatically. Mutual MyCloud device certificate authentication before connection. WebRTC DTLS-SRTP encrypted stream. Maximum 1080p30 on 5GHz Wi-Fi, 1080p60 on Ethernet. Note: MyPhone uses Wi-Fi 6E (802.11ax); MyTV uses Wi-Fi 5 (802.11ac). A dual-band router supporting 802.11ac at 5GHz or better is required for reliable 1080p30. Screen casting activates the MyPhone seL4 wallet lockout (see MyPhone Architecture Roadmap Section 19.10) — MyPhone wallet transaction signing is architecturally impossible while casting is active. | Local Wi-Fi (same network). No cloud relay. |
| Matrix call handoff | "Hey MyTV, take the call." An active MyPhone Matrix E2EE call transfers to MyTV's Leopard camera and room audio. MyTV joins the Matrix room with its own device identity. E2EE preserved throughout. MyPhone remains in the room as a second participant. | Local Wi-Fi or internet — Matrix protocol works either way. |
| Shared media library | Tier 2: MyPhone Cast serves its on-device library via encrypted HTTPS (TLS 1.3, mutual MyCloud device cert auth) to MyTV when MyPhone is on the local network. Tier 3: when MyPhone is away, MyTV accesses MyCloud-synced media directly — no MyPhone required. MyTV authenticates to MyCloud with its MTV- device certificate and decrypts using the Layer A MyCloud Account Key. | Tier 2: local Wi-Fi. Tier 3: internet. |
| Phone-as-remote | MyPhone companion app (MyTV Bridge, available in MyCloud App Store) connects to MyTV's remote API via mDNS discovery and mutual MyCloud device certificate authentication. Provides d-pad, keyboard, touchpad, media transport controls, and voice passthrough. The companion app runs as a Flatpak sandbox in MyPhone's Linux VM with no access to the seL4 TCB. See MyPhone Architecture Roadmap Section 9.8. | Local Wi-Fi only. |
| Presence profile unlock | MyTV detects registered MyPhone devices via the MyPhone mDNS presence beacon (_mypresence._udp service, opt-in — see MyPhone Architecture Roadmap Section 9.7) with approximately 10 seconds of detection latency. When the owner's phone is detected on the network, MyTV silently switches to the owner's profile. Multiple family members each get their own profile activated when their phone is home. When MyPhone leaves the network, the active profile remains until another registered device is detected or the user manually switches. Presence detection uses the MyPhone mDNS presence beacon (_mypresence._udp, opt-in per device — see MyPhone Architecture Roadmap Section 9.7). | Local Wi-Fi only. |
| Multiplayer invitations | RetroArch Netplay host addresses are sent as E2EE Matrix messages via MyCloud to known contacts. No separate invite system. The recipient's MyTV or MyPhone companion app auto-connects on acceptance. | Matrix (local or internet). |

# 10. MyCloud — Subscription Integration

MyTV operates fully standalone for media, gaming, and workstation functions without any account or subscription. The MyCloud subscription unlocks encrypted communication, cloud media sync, cross-device continuity, and MyCloud App Store access. The subscription is priced at a single flat rate covering all MyCloud services across all enrolled devices — see current pricing at mycloud.com. A user who owns only a MyTV (with no MyPhone) has full access to all MyCloud features relevant to their device.

| Feature | Without subscription | With MyCloud subscription |
| --- | --- | --- |
| MyTV Media Center — local and USB media | Full access | Full access |
| Voice control — all agents | Full access | Full access |
| Retro Games — RetroPie | Full access | Full access |
| RetroArch Netplay (local Wi-Fi) | Full access | Full access |
| Browser | Full access | Full access |
| Display input switching | Full access | Full access |
| Cross-product: screen cast, phone-as-remote, presence unlock | Full access (local Wi-Fi) | Full access |
| Matrix E2EE video calling (1:1 and group) | Not available | Full access via MyCloud Matrix service (matrix.mycloud.com). Zero-day metadata retention. MyTV registers as a Matrix device directly with MyCloud using its device certificate. See MyCloud Architecture Roadmap Section 6. |
| MyPhone media library sync (Tier 3 cloud) | Not available | Photos and documents auto-sync. Audio and video user-selected playlists. Layer A client-side E2EE (MyCloud Account Key — no MyPhone required for MyTV to decrypt). See MyCloud Architecture Roadmap Section 5. |
| Encrypted settings and profile backup | Not available | MyTV settings, profiles, and media library metadata backed up to MyCloud as an encrypted blob. Layer A encrypted (MyCloud Account Key). Accessible from any enrolled device. |
| OTA updates | Via public MyTV OTA repository | Via MyCloud CDN — same distribution network used for MyPhone firmware. |

The subscription model means the MyTV hardware can be sold as a standalone product — users benefit immediately from local media, gaming, and voice control without any account requirement. The subscription provides the communication and cloud layer for users who want it, creating a recurring revenue stream from the same monthly plan that covers their MyPhone services.

# 11. Development Roadmap

## Phase 0 — Validation Sprint (Weeks 1–2)

Proof-of-concept validation before committing to production hardware. All targets must be met before Phase 1 begins.

- Gemma 2 2B IT INT4 on Pi 5: download GGUF file, run llama.cpp NEON benchmark. Target: >6 tokens/sec.

- Hailo-8L USB3 mode + Whisper Small INT8: end-of-utterance to transcript. Target: <450ms.

- PipeWire WebRTC AEC with Yeti Nano: TV audio at 70dB, openWakeWord false trigger rate. Target: <0.1 per hour.

- Rode Wireless ME echo rejection at 3m room scale: same false trigger rate target.

- Infineon SLB9670 via SPI GPIO: tpm_tis_spi driver load, systemd-cryptenroll LUKS enrollment, PCR-bound unlock verified.  MyCloud device provisioning: TPM2_CreatePrimary Attestation Key generated, TPM2_Certify attestation quote produced, submitted to MyCloud Device Provisioning endpoint, MyCloud device certificate (MTV- namespace) issued and stored in TPM-sealed storage. End-to-end provisioning flow validated. This is a Phase 0 blocking gate — MyCloud Device Provisioning CA must be operational (MyCloud Phase 0) before MyTV Phase 0 can complete.

- CEC input switching to NEC M-series display: "switch to HDMI 2" command validated.

- MyPhone Cast prototype: mDNS advertisement (_mycast._tcp), TLS 1.3 mutual MyCloud device cert handshake, HTTP/2 Range request video seek. Target: <100ms first frame on local 5GHz Wi-Fi.

- RetroArch Netplay on Pi 5: Netplay session established between two devices (MyTV + MyTV and MyTV + MyPhone companion app). Input latency measured on LAN. Target: <10ms round-trip on local Wi-Fi.

## Phase 1 — Foundation (Weeks 3–8)

- Raspberry Pi OS Bookworm 64-bit base build. Labwc Wayland compositor. TV-mode 10-foot shell.

- PipeWire audio stack with AEC module configured for both Yeti Nano and Rode Wireless ME.

- Hailo-8L USB3 Whisper pipeline. openWakeWord integration. End-to-end voice pipeline validated.

- MiniLM-L6 fine-tuned intent classifier (training dataset: 5,000 TV command utterances + 2,000 negative examples). Gemma 2 2B IT INT4 fallback integration.

- Infineon SLB9670 hardware TPM + LUKS disk encryption + PCR binding. A/B partition OTA layout on NVMe. Suspend-to-RAM.

- MyPhone Cast server (production): full media library API, WebSocket events, thumbnail serving, mutual MyCloud device certificate authentication (MP- namespace MyPhone cert verified against paired device list).

- MyTV Media Center source router: USB, Cast, and Cloud tiers. Source indicator in UI.

- CEC input switching and nftables firewall configuration.

## Phase 2 — Applications (Weeks 9–18)

- MyTV Media Center: all four media pillars, unified SQLite library, 10-foot browser UI, yt-dlp integration.

- Conference: native Matrix client (Matrix Rust SDK), D-Bus interface for Conference Agent, 1:1 VoIP, Element Call group calls, CalDAV integration, call handoff from MyPhone.

- Games: RetroPie + EmulationStation, RetroArch Netplay (Mode 2: Play Together), Games Agent voice command routing, Matrix E2EE multiplayer invitation flow. MyPhone companion app Netplay client integration validated end-to-end.

- Browser: Chromium TV mode (VP9 forced) and Workstation mode.

- MyCloud media sync: photos and documents auto-sync pipeline, audio/video user-selection UI, client-side encryption via SPU, MyTV decrypt flow.

- Initial setup flow: language selection, Wi-Fi configuration, MyPhone pairing, display calibration, media source setup.

## Phase 3 — Cross-Product Integration (Weeks 19–24)

- Screen cast (WebRTC, 1080p30, mutual MyCloud device cert auth). MyPhone companion app (MyTV Bridge) available in MyCloud App Store — provides screen cast initiation, phone-as-remote, presence toggle, RetroArch Netplay client for Mode 2. Validated end-to-end against MyPhone Architecture Roadmap Section 9.8 specification.

- Matrix call handoff end-to-end test with E2EE verification. Presence-based profile unlock.

- Pairing revocation flow: local Settings UI and remote revocation via MyCloud.

- Multiplayer invitation via Matrix E2EE: Netplay host addresses delivered to contacts. MyTV + MyTV and MyTV + MyPhone companion app cross-device Netplay tested end-to-end with MyCloud relay service.

## Phase 4 — Launch Preparation (Weeks 25–30)

- User testing with 15 households. Focus areas: voice command coverage, MyPhone Cast discovery reliability, Matrix call quality via MyCloud, Netplay cross-device latency (MyTV + MyTV and MyTV + MyPhone companion app), MyCloud provisioning and device enrollment flow.

- Manufacturing setup: NVMe pre-image, TPM attestation key generation at first boot, MyCloud Device Provisioning endpoint integration validated at scale (all devices must receive MTV- namespace certificates), factory acceptance test including AEC validation for both Yeti Nano and Rode Wireless ME configurations.

- Documentation: user quick-start guide, microphone installation guide, MyPhone pairing guide.